Work Plan

Borehole Backfilling

Within

Chicopee Falls Levee System

Right-of-Way

Former Facemate Corporation Facility

5 West Main Street

Chicopee, Massachusetts 01020

September 20, 2010

Prepared for Submittal to:

United States Army Corps of Engineers

696 Virginia Road

Concord, MA 01742-2751

Prepared on Behalf of:

City of Chicopee

Office of Community Development

38 Center Street

Chicopee, Massachusetts  01020

Prepared by:

Table of Contents

1.0	Introduction	1

2.0	Summary of Work Completed to Date	2

3.0	Corrective Borehole Backfilling Activities	2

4.0	As-Built Plan	3

Figures

1	Site Locus Map

2a	Site Plan: U.S. Army Engineer Division Drawing Number CT-5885 Sheet 2

2b	Site Plan: Blow-up of Portion of Figure 2b

Appendices

A	Soil Boring Logs

# 1.0	Introduction

On behalf of the Chicopee Office of Community Development (OCD), BETA Group, Inc. (BETA) has prepared this Work Plan for the backfilling of up to two boreholes recently installed within or near the right-of-way of the Chicopee Falls Levee System at the former Facemate Facility property located at 5 West Main Street in Chicopee, Massachusetts (the Site).  It is also intended that this work plan will be the initial step in defining an established procedure for communicating and coordinating future activities within that right-of-way, as well as obtaining any required advance approval of those activities from the interested parties with jurisdiction.

BETA is under contract with the City of Chicopee to provide Licensed Site Professional and related services in to identify and assess environmental conditions at the Former Facemate Facility, as well as technical and regulatory compliance assistance relative to planning for future development of the Site.  BETA will also be providing remediation (and clean-up) activities.

BETA understands that the United States Army Corps of Engineers (USACE) maintains jurisdiction over, and oversees operation and maintenance of, the Chicopee Falls Levee System.  The Chicopee Falls project provides flood protection for the highly industrialized area of Chicopee along the right (north) bank of the Chicopee River about two miles upstream from the mouth of the river (Chicopee Falls). The project consists of about 4,800 feet of levees and walls, two pressure drains for discharging high level interior drainage, and two pumping stations for industrial process water.  The Chicopee Falls project was built following the Chicopee River flood of August 1955.  Construction was started in October 1963 and completed in July 1965.

Additionally, BETA is now aware of the levee system, USACE’s jurisdiction over the levee system, and that approval from USACE is required prior to conducting certain activities within the levee right-of-way.

Numerous sources of both oil and hazardous material have been present at the Site property throughout its industrial history, including large capacity underground storage tanks containing #6 fuel oil, above ground storage tanks containing hydrogen peroxide and muriatic acid, and both non-PCB and PCB-containing transformers.

Limited site investigations have been conducted by others at the Site to date.  BETA is undertaking supplemental investigations to more fully assess the nature and extent of environmental conditions at the Site that may represent significant risk to human health, safety and/or the environment in the present, or under potential future uses.  Those supplemental investigation activities include, among other things, subsurface explorations (drilling of soil borings, installation of monitoring wells, excavation of test pits, etc.), some of which will need to be within or in close proximity to right-of-way of the Chicopee Falls Levee System.  As of the date of this Work Plan, BETA subsurface investigation has occurred at only one (perhaps two, depending on USACE’s interpretation) location that is immediately proximate to the levee.

The full extent and location of remediation activities is not currently known, but will be dependent on the findings of the ongoing investigations.  BETA is currently in the design phase of a project to remove underground storage tanks (USTs) located in the inner (northwest) corner of the “elbow” formed by Buildings 1 and 5 as shown on the USACE Drawing Number CT-5885 Sheet 2 (June 1963 Corps Chicopee Falls Flood Control Plans).  There are three USTs to be removed, none of which is adjacent to the levee; the nearest tank to the levee is approximately 70 feet from the 24-inch R.C.P. interceptor drain.  Despite the buffer distance to the levee, the Contract Documents for the UST removal project will include notification to the contractor of the levee system, define a limit of work that provides a separation from the levee system (including the interceptor drain), and stipulate that the contractor’s activities are strictly prohibited beyond that limit of work.

# 2.0	Summary of Work Completed to Date

BETA was on Site on August 26, 2010 to complete soil borings as part of the on-going supplemental investigation.  Two of these borings, SB-10 and SB-11, were completed at the western side of the property; SB-10 is approximately 50 feet east of the 18-inch R.C.P. interceptor drain, and SB-11 is approximately five (5) feet east of the 18-inch R.C.P. interceptor drain.  A Site Locus is included as Figure 1 and Site Plans are included as Figures 2a and 2b (see figure 2a for boring locations).  The borings were drilled using 4-3/8 inch (ID) hollow stem augers with a 140 pound hammer to drive 1-3/8 inch split barrel sampler.  Boring SB-10 was advanced to fourteen (14) feet below ground surface (bgs) and boring SB-11 was advanced to eighteen (18) feet bgs.  Though both borings were backfilled upon completion, the method of backfilling did not meet USACE requirements.

Soil boring logs are included in Appendix A.

3.0	Corrective Borehole Backfilling Activities

BETA proposes to re-drill soil boring SB-11 (and potentially SB-10, if required by USACE) and then backfill it in accordance with USACE Regulation ER 1110-1-1807 (Procedures for Drilling in Earth Embankments).  Grout conforming to the requirements of USACE Engineering Manual EM 1110-2-3506 (Grouting Technology) will be injected through a tremie pipe or hose inserted to a depth of six (6) inches above the base of the boring.  The hole will then be backfilled (grouted) to twelve (12) inches below the ground surface.  The quantity of backfill grout will be estimated based on the depth and diameter of each bore hole prior to starting backfilling operations, and injection quantities will be monitored continuously.  If the estimated quantity per linear foot of hole is exceeded by a meaningful amount at any time, operations will be halted, and the casing will be pulled to the top of the grout and the backfill allowed to set.  Prescribed backfilling operations will then resume.

# 4.0	As-Built Plan

The re-drilling and grouting of each borehole will be logged; the logs and an as-built plan showing the locations of the grouted holes will be prepared and submitted to USACE following completion.

Figures

Appendix A

Boring Logs

The Site Plans for this Work Plan are based on the drawings entitled “Connecticut River Flood Control Project, Chicopee Falls, Mass., Plans for the Local Protection Project”, dated June 1963 (U.S. Army Engineer Division, New England, Corps of Engineers, Waltham, Mass.).  Figure 2a is a reduction of Drawing Number CT-5885 Sheet 2, scaled to fit an 11 inch by 17 inch sheet.  Figure 2b is a blow-up of a portion of Figure 2b; the area of coverage of Figure 2b is shown on Figure 2a.

The Site Plans for this Work Plan are based on the drawings entitled “Connecticut River Flood Control Project, Chicopee Falls, Mass., Plans for the Local Protection Project”, dated June 1963 (U.S. Army Engineer Division, New England, Corps of Engineers, Waltham, Mass.).  Figure 2a is a reduction of Drawing Number CT-5885 Sheet 2, scaled to fit an 11 inch by 17 inch sheet.  Figure 2b is a blow-up of a portion of Figure 2b; the area of coverage of Figure 2b is shown on Figure 2a.